Wyuna progress report from 18-21 February

Quick turnaround, back in Tassie as the former Wyuna chief engineer Oboy will be going back to sea the following week. Fortunately in perfect timing I was contacted by Chris Ewbank. Chris, who was one of the former Wyuna chief engineers made an offer of help. Chris like Oboy is still at sea and was on leave for several weeks, Oboy will be back end of March so the rotation works well.

Our Boson Little Max, and myself went on a hunt to try and locate some of Wyuna’s missing items. Low Head museum apparently have the fog horn but they were playing it cool and not volunteering any info. We went to Georgetown maritime museum and they immediately stated they had an 18’ Wyuna skiff and we could have it back. Lovely people and a great museum, if you are in Tassie, their Georgetown museum is well worth a visit.

With these two talented engineers on the job we have made massive inroads in to the technical work. In the race to get ready for the engineers to run engines we have removed cleaned and restored and reinstalled 252 brushes in the generators. Armatures cleaned again, some sanded with very fine sandpaper to remove the scale then washed with white spirits.

Propulsion motor

Main generators

Then after working huge hours over a number of days the moment came as the engineers’ ran up air. Unfortunately we did cease the Cummins generator as it was never designed to run at high speeds normally it would be bolted to a slow revving diesel. As reported last issue the two DC generators were started as well as No2 main engine. However, only No4 gen set was on the board producing power. This time the proposal was, we would run No2 and activate the generator run the exciters and run the propulsion units.

No2 main engine running, here is Chris livening up the electrical panel for the propulsion system which has not run since 2004. Chris was amazed that it all fired up okay and still worked after such a layup.

Note the two wheels that control the power going to the propulsion motors which inturn control the shaft revolutions. Note the lights for forward or astern, shown in neutral position. The light can reflect commands from the bridge and the wheels turn automatically or can be run manually from the engine room

Difficult to see but this shot is taken over the stern with the propellers slowly turning. This is a huge milestone, if you were not involved it is difficult to grasp the amount of work it took to get to this point. I congratulate all those that have worked tirelessly to make this happen an amazing achievement to put life back in to a ship.

Stop Press Chris has informed me all main engines now running, steering gear operational engine bridge controls now also operational No 5 DC generator on the board producing power. Great News